WETLAND & WATERSHED PERMITS CHECKLIST

Date:	Project Address:

WATERSHED PROTECTION OVERLAY DIST. (WPOD): SITE PLAN/SUBDIVISION

Site Diagram/Sketch/Plot Plan depicting the existing and proposed development including all impervious surfaces (driveways, walkways, boat ramps, decks, etc.). Include total square footage of each impervious surface. Roofs are excluded from the impervious calculation, provided that clean rainwater is transmitted to a recharge system, meeting NH DES requirements.

2 copies of a hydrologic study (see Section 616.7). The Hydrological study must document, in a manner acceptable to the Planning Board, that the proposed land development would provide the same or greater degree of water quality protection as existed on the site (s) at the time the application was made. Change of Use Applications that do not propose any new construction, paving, alterations to grading, or other alteration to the terrain are exempt from the requirements of the hydrological study.

Information, on submitted Plans or in a written narrative, as to how the following criteria have been met. If “not applicable” provide written notice as such:

616.6.4 - All development within the Watershed Protection Overlay District shall be evaluated to ensure that:

616.6.4.1 - No new impervious driveways are allowed within 75 feet of any surface water or wetland area. Accessory structures are allowed when permitted by the NH DES.

616.6.4.2 - The impervious surface of any lot is limited to 30%. For lots that currently exceed 30% impervious surface, development must decrease the percent of impervious surface. Replacement in-kind of existing development does not require this reduction of impervious surface.

616.6.4.3 - Non-point source pollution is prevented to the maximum extent possible, taking into account site conditions such as slope, soil type and erosivity and vegetative cover.

616.6.4.4 - Best Management Practices (BMPs) are in place and are sufficient to remove or neutralize those pollutants that present a potential impact to the water body. The use or creation of detention ponds is not allowed for runoff control, except in those cases where an extended detention pond may be necessary to develop a site.

616.6.4.5 - Grading and removal of vegetation at a development site is minimized and erosion and sedimentation control measures are in place and properly installed.

616.6.4.6 - If two or more dwelling units share a common sewage treatment system a perpetual maintenance agreement from the building’s owner is required.

616.6.4.7 - Uses that may potentially cause contamination within the Watershed Protection Overlay District, must submit a spill prevention control and countermeasures plan for approval. This plan shall include the following elements:

616.6.4.7.1 - Disclosure statements describing the types, quantities, and storage locations of all contaminants that will be part of the proposed project.

616.6.4.7.2 - Contaminant handling and spill prevention techniques.

616.6.4.7.3 - Spill reporting procedures, including a list of affected agencies to be contacted in the event of a spill.

616.6.4.7.4 - Spill recovery plans, including a list of available equipment.

616.6.4.7.5 - Spill cleanup and disposal plans.

616.7.1 - A hydrologic study shall be done by a professional engineer or hydrologist licensed in the State of New Hampshire and shall include the following information:

616.7.1.1 - Description of the proposed project including location and extent of impervious surfaces; on-site processes or storage of materials; the anticipated use of the land and March 2016 buildings; description of the site including topographic, hydrologic and vegetative features.

616.7.1.2 - Characteristics of natural runoff on the site and projected runoff with the proposed project, including its rate and chemical and/or biological characteristics deemed necessary to make an adequate assessment of water quality.

616.7.1.3 - Measures proposed to be employed to reduce the rate of runoff and pollutant loading of runoff from the project area, both during construction and after.

616.7.1.4 - Proposed runoff control and watershed protection measures for the site. These measures shall be designed with the goal of ensuring that the rate of surface water runoff from the site does not exceed pre-development conditions and that the quality of such runoff will not be less than pre-development conditions. Special emphasis shall be placed on the impacts of proposed encroachments into the required buffer.

616.7.1.5 - Where the developer of property subject to the terms of this Watershed Protection Ordinance seeks to utilize existing or planned off-site storm-water quality management facilities, the developer shall provide a written certification that the owner of the off-site facilities will accept the runoff and be responsible for its adequate treatment and that the arrangement will run with the land in a manner that will be acceptable to the Planning Board.

WATERSHED PROTECTION OVERLAY DIST. (WPOD): MAJOR

7 copies of a Site Diagram/Sketch/Plot Plan depicting the existing and proposed development including all impervious surfaces (driveways, walkways, boat ramps, decks, etc.). Include total square footage of each impervious surface.

2 copies of a storm water management and erosion control plan, consistent with New Hampshire Stormwater Manual (latest edition) prepared by NH DES. This shall be prepared and submitted for review and approval as part of a Major Watershed Application unless previous approval has been granted through the Site Plan/Subdivision Watershed Application process. Major Watershed Applications are reviewed and approved by the Planning Board (see Section 616.6.1.2).

616.6.4 - All development within the Watershed Protection Overlay District shall be evaluated to ensure that:

616.6.4.1 - No new impervious driveways are allowed within 75 feet of any surface water or wetland area. Accessory structures are allowed when permitted by the NH DES.

616.6.4.2 - The impervious surface of any lot is limited to 30%. For lots that currently exceed 30% impervious surface, development must decrease the percent of impervious surface. Replacement in-kind of existing development does not require this reduction of impervious surface.

616.6.4.3 - Non-point source pollution is prevented to the maximum extent possible, taking into account site conditions such as slope, soil type and erosivity and vegetative cover.

616.6.4.4 - Best Management Practices (BMPs) are in place and are sufficient to remove or neutralize those pollutants that present a potential impact to the water body. The use or creation of detention ponds is not allowed for runoff control, except in those cases where an extended detention pond may be necessary to develop a site.

616.6.4.5 - Grading and removal of vegetation at a development site is minimized and erosion and sedimentation control measures are in place and properly installed.

616.6.4.6 - If two or more dwelling units share a common sewage treatment system a perpetual maintenance agreement from the building’s owner is required.

616.6.4.7 - Uses that may potentially cause contamination within the Watershed Protection Overlay District, must submit a spill prevention control and countermeasures plan for approval. This plan shall include the following elements:

616.6.4.7.1 - Disclosure statements describing the types, quantities, and storage locations of all contaminants that will be part of the proposed project.

616.6.4.7.2 - Contaminant handling and spill prevention techniques.

616.6.4.7.3 - Spill reporting procedures, including a list of affected agencies to be contacted in the event of a spill.

616.6.4.7.4 - Spill recovery plans, including a list of available equipment.

616.6.4.7.5 - Spill cleanup and disposal plans.

WETLAND AND WATERSHED PROTECTION (WWPD) DIST. SPECIAL PERMIT

601.4.8.1 - The following statement must be entered on the plan: NOTE: No structure may be erected nor shall any alteration of the surface configuration of the land be permitted in the Wetland and Watershed Protection District, except as authorized by this approval.

601.4.8.2 - The plan shall also indicate: the location and limits of the proposed activity; the construction techniques and sequence to be used in constructing the proposed improvements; and the protective measures to be employed to minimize the disturbance and/or degradation of the WWPD.

601.4.8.3 - The applicant shall also submit evidence [a separate letter] that: the WWPD disturbance is the minimum necessary to affect the proposed improvements; the proposed activity will not contribute to the degradation of surface or groundwater quality; the proposed activity is appropriate and safe to locate in the WWPD; and, the proposal is consistent with the intent and purpose of Section 601 of the Windham Zoning Ordinance.

Plans showing profiles, cross-sections, and elevations at 50-foot maximum intervals, for any proposed street, drives, access ways, or other disturbance of the WWPD;

Two (2) sets of color photographs (of minimum 4” x 6” size) taken in both directions along the centerline of any proposed street or access way at intervals corresponding with the intervals delineated on the plans submitted in accordance with 601.4.8.3a

If the plan is part of a Site Plan or Subdivision application the required items indicated above may be included with the main plan set for site plan or subdivision. If this application is not being included with a Site Plan application or Subdivision application, include all items indicated above with your application.

601.4.8.4 - WWPD Boundary Markers Required: Where any activity requiring Planning Board approval is proposed within the WWPD, the WWPD boundaries on such parcel (or such lesser area as the Planning Board may specify) shall be marked in order to prevent encroachment in accordance with Section 601.4.8.4. Per 601.4.8.4, the cost of the purchase of the WWPD boundary markers from the Community Development Department and the placement of them by a certified wetland scientist shall be borne by the applicant/developer or their successors in interest.

Upon the Approval of an Application by the Planning Board, Staff will provide you with the total amount due for WWPD Boundary Markers (per adopted fee schedule), which is to be paid when picking up the markers from the Community Development Department.

Owner’s Concurrence – The owner concurs in the representation by the agent and agrees that the information presented is accurate, agrees to the Windham Planning Board review process, and requests Planning Board approval of the above identified Subdivision Plan.

_______________________________________________________________________

Owner’s Signature 				Date

Agent’s Request – The Agent seeking Planning Board review of the attached information represents his information presented by the Agent is to the best of his/her knowledge in accordance with the Town of Windham Zoning Ordinance and Subdivision Regulations and all revisions thereof.

________________________________________________________________________

Agent’s Signature 				Date